| Hazard | Failure Mode | Causes | Consequences | Risk Priority Rating |  |  |  | Mitigation | Risk Priority Rating |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Sev | Prob | Det | RPN |  | Sev | Prob | Sev | Prob |
| Battery | Fails to provide adequate power | Due to weak battery | Fails to operate (no movement) | 4 | 2 | 3 | 24 | Change of battery type | 1 | 1 | 2 | 4 |
|  | Loss of containment of lithium (Exposure to atmosphere) | Defective battery | Lithium fire, Explosion | 2 | 2 | 2 | 8 | Change of battery type | 2 | 1 | 1 | 2 |
|  |  | Damage to casing | Lithium fire, Explosion | 4 | 2 | 1 | 8 | Thicker/stronger casing material Secondary containment Change battery type Annual inspection/maintenance | 3 | 1 | 1 | 3 |
|  | Short circuit | Defective battery | Motor stops, No movement | 2 | 2 | 4 | 16 | Secondary containment Change battery type Annual inspection/maintenance | 3 | 1 | 2 | 6 |
|  |  | Faulty wiring | Motor stops, No movement | 2 | 2 | 2 | 8 | Annual inspection/maintenance Simplify writing | 2 | 1 | 2 | 4 |
|  | Inability to hold a charge | Lithium expansion (worn battery) | Can’t be charged, No movement | 2 | 3 | 2 | 12 | Annual inspection/maintenance Annual battery replacement Change of battery type | 2 | 1 | 2 | 4 |
| Servomotor | Abnormal noise | High voltage power supply | Abnormal noise made by the prototype | 3 | 2 | 2 | 12 | Best selection of servo motor | 2 | 1 | 1 | 4 |
| Senor System (Joystick) | Abnormal vibration | Uneven air gap | Abnormal vibration made by the prototype | 1 | 2 | 2 | 4 | Best selection of servo motor | 2 | 1 | 1 | 2 |
|  | Incorrect feedback | Due to coding failure | Incorrect prototype motion | 3 | 2 | 2 | 12 | Change of senor system | 3 | 1 | 2 | 6 |
| Control System | Uncontrolled current | Power supply failure | Incorrect prototype motion | 3 | 2 | 2 | 12 | Change of circuit board | 4 | 1 | 2 | 8 |
|  | Continues previous motion | Circuit failure | Incorrect prototype motion | 3 | 1 | 2 | 6 | Change of circuit board | 4 | 1 | 1 | 4 |
| Switch | Fails to open | Defective switch | Affect the performance of prototype | 1 | 2 | 4 | 8 | Replacement of switch | 2 | 1 | 2 | 4 |
| Cables | Damaged caused to cable | The circuit does not work properly | No proper engineering design | 3 | 2 | 2 | 12 | Designing the prototype in a proper manner | 1 | 1 | 1 | 1 |
| Chemical Failure | Fluid inside the tank is not effective against the fire | Failure to eliminate the fire on time | No proper selection of fluid type | 4 | 2 | 1 | 8 | Selection of fluid based on its performance type | 2 | 1 | 2 | 4 |
| Wheels | Entanglement | Introduction of foreign objects | Loss of control over the prototype | 2 | 2 | 4 | 16 | Solid wheels from strong material. | 3 | 1 | 2 | 6 |
|  | Wheel failure | Rupture/chipping | Damage to prototype | 2 | 4 | 1 | 8 | Selecting best wheels for the prototype | 2 | 1 | 1 | 2 |
| Gearbox | Gearbox failure | High loading in prototype | Loss of control over the prototype | 3 | 2 | 1 | 6 | Having restriction for overload | 1 | 1 | 1 | 1 |
| Gimbal | Gimbal motor failure | Defective gimbal | Inability to move in different axis | 4 | 4 | 1 | 16 | Replacement of gimbal | 3 | 1 | 2 | 6 |